Here is the comprehensive documentation of the details and formulas based on the research article1.

1. Solar PV Panel Technical Specifications

The calculations rely on the specific characteristics of the 10W panels used in the study2.

| Parameter | Symbol | Value (at STC) |
| --- | --- | --- |
| Maximum Power | $P_m$ | 10 W |
| Maximum Power Voltage | $V_m$ | 20 V |
| Maximum Power Current | $I_{im}$ | 0.50 A |
| Short Circuit Current | $I_{sc}$ | 0.60 A |
| Open Circuit Voltage | $V_{oc}$ | 24.8 V |
| Temp. Coefficient ($P_m$) | $\beta_p$ | $-0.3\%/^{\circ}C$ to $-0.5\%/^{\circ}C$ |
| Temp. Coefficient ($I_{im}$) | $\alpha_{Im}$ | $+0.1\%/^{\circ}C$ |
| Temp. Coefficient ($V_m$) | $\beta_{Vm}$ | $-0.38\%/^{\circ}C$ |

2. Core Mathematical Formulas

A. Cell Temperature ($T_{cell}$)

The real-world temperature of the solar cell is calculated based on ambient conditions and irradiation4.

$$T_{cell} = T_{\infty} + 0.078 H_{g}$$

- $T_{\infty}$: Ambient air temperature ($^{\circ}C$)5.

- $H_g$: Global inclined solar irradiance ($W/m^2$)6.

B. Real Power Output ($P_{pv}$)

This formula estimates power under real operation and climatic conditions7.

$$P_{pv} = P_{stc} \times DF_{pv} \times (H_g / H_{stc}) \times [1 + \beta_p(T_{cell} - T_{stc})]$$

- $P_{stc}$: Nominal PV power at STC (10 W)8.

- $H_{stc}$: Standard solar radiation (1000 $W/m^2$)9.

- $T_{stc}$: Standard test temperature (25 °C)10.

- $DF_{pv}$: PV derating factor11.

C. Output Current ($I$) - Single Diode Model

The standard electrical behavior of the PV cell is represented by this equation12.

$$I = I_{ph} - I_0 \left( e^{\frac{q(V + I R_s)}{n k T}} - 1 \right) - \left( \frac{V + I R_s}{R_{sh}} \right)$$

- $I_{ph}$: Photocurrent13.

- $I_0$: Diode saturation current14.

- $R_s$ / $R_{sh}$: Series and Shunt resistances15.

- $n$: Ideality factor16.

D. Fill Factor ($FF$)

This represents the ratio of maximum power to the theoretical power17171717.

$$FF = \frac{P_m}{V_{oc} \times I_{sc}}$$

3. Array Configuration Scaling (Se-P Topology)

The experiment utilized a 12-panel Series-Parallel (Se-P) array in a $3 \times 4$ arrangement181818. To calculate the final array values, you must scale the single-panel results:

- Array $V_{oc}$: $V_{oc,panel} \times 3$ (Number of panels in series)19.

- Array $I_{sc}$: $I_{sc,panel} \times 4$ (Number of strings in parallel)20.

- Array $V_m$: $V_{m,panel} \times 3$21.

- Array $I_m$: $I_{m,panel} \times 4$22.

- Array $P_m$: $V_{m,array} \times I_{m,array}$23.

4. Experimental Positioning Details

The study highlights that the output power depends heavily on the variance (mounting height) and positioning24242424.

- Flat Position (FP): Panels placed horizontally at 180°25.

- Highest Performance Setup: Achieving 156 W with a flat-position array at a 93 cm variance using Se-P interconnection26.

- Footprint Optimization: While FP yields more power, V-shape (VP) and Inverted V-shape (IVP) are strategic for reducing the land footprint in urban environments272727272727272727.

code 

import csv

from datetime import datetime, timedelta

import math

import random

def generate_summer_pv_csv(

start_date="2025-04-01",

months=3,

output_file="pv_summer_apr_may_jun.csv"

):

# ===============================

# SYSTEM CONSTANTS (from paper)

# ===============================

FF = 0.67

Voc_stc = 24.8

Isc_stc = 0.60

T_stc = 25

n_series = 4

n_parallel = 3

beta_voc = -0.0038     # −0.38 % / °C

alpha_isc = 0.001      # +0.1 % / °C

current_loss_factor = 0.82

voltage_factor = 0.84

vm_ratio = 0.80

# Time slots: 08:30 – 18:30 (summer longer sun)

time_slots = [

(datetime.strptime("08:30", "%H:%M") + timedelta(minutes=30*i)).strftime("%H:%M")

for i in range(21)

]

start_dt = datetime.strptime(start_date, "%Y-%m-%d")

end_dt = start_dt + timedelta(days=months * 30)

with open(output_file, mode="w", newline="") as file:

writer = csv.writer(file)

writer.writerow([

"Date", "Time", "Irradiation_Wm2", "AmbientTemp_C", "CellTemp_C",

"Voc_V", "Isc_A", "Vm_V", "Im_A", "Pm_W", "FF"

])

current_date = start_dt

while current_date < end_dt:

# Summer ambient temperature

T_ambient = random.uniform(32, 42)

# Higher summer peak irradiation

daily_peak = random.randint(1050, 1150)

for idx, time_str in enumerate(time_slots):

# Summer irradiation curve (wider bell)

angle = (idx / (len(time_slots) - 1)) * math.pi

irradiation = max(

250,

daily_peak * math.sin(angle) + random.uniform(-40, 40)

)

# Cell temperature (summer realistic)

T_cell = T_ambient + (0.025 * irradiation / 10)

delta_T = T_cell - T_stc

# -------------------------------

# ELECTRICAL CALCULATIONS

# -------------------------------

Isc_panel = (

Isc_stc *

(irradiation / 1000) *

(1 + alpha_isc * delta_T) *

current_loss_factor

)

Isc_array = Isc_panel * n_parallel

Voc_panel = (

Voc_stc *

voltage_factor *

(1 + beta_voc * delta_T)

)

Voc_array = Voc_panel * n_series

Vm_array = vm_ratio * Voc_array

Pm_array = FF * Voc_array * Isc_array

Im_array = Pm_array / Vm_array if Vm_array > 0 else 0

writer.writerow([

current_date.strftime("%Y-%m-%d"),

time_str,

int(irradiation),

round(T_ambient, 1),

round(T_cell, 1),

round(Voc_array, 2),

round(Isc_array, 2),

round(Vm_array, 2),

round(Im_array, 2),

round(Pm_array, 0),

FF

])

current_date += timedelta(days=1)

print(f"✅ Summer CSV generated: {output_file}")

# RUN

generate_summer_pv_csv()